Glenn Smith Sound & Vision

6 Garden Court, Berwick 3806

Mobile - 0412598338

Email – smithonline@netspace.net.au

Health and Safety Guide for Workers.

1. TRUCK UNPACK

Truck	- Crew to guide reversing truck where possible. Stand behind driver side mirror at

rear of truck in vision of driver. Do not stand behind truck out of sight of driver.

- Care must be taken opening the rear truck doors. Watch out for people, cars and obstacles

Hydraulic Tailgate	- Only G.S.S.V. crew to operate the tailgate.

- Full concentration required during operation.

- Care must be taken to keep clear of  gaps between tailgate and truck body.

- Joy riding on tailgate is forbidden.

- (Failure to follow tailgate rules will incur severe warning or termination of  employment.)

Loading ramp		- 2 crew to lift ramp where possible.

- Ensure locking pin is in place.

- No stacking gear on cases allowed on ramp.

- 2 crew on each case where ramp angle is steep.

- No running or jumping on ramp at any time.

2. LOADING IN

- All heavy road cases are on casters.

- Trolley available for subs, Desks. etc.

- Lighter gear may be stacked in a safe manner on road cases. No overloading.

- Min 2 crew to lift cases above 300mm. Min 3 crew to lift cases up stairs. Use  proper technique.

i.e. Bend knee, back straight.

- Use 2 crew to carry staging sections where possible.

- 2 crew to push cases up or down steep incline.

- Appropriate footwear must be worn. i.e. no thongs or sandals

-.Closed in shoes only.

- Walking pace only when pushing cases.

- Take extra care around other people.

- Drink adequate fluids in warmer weather.

3. SETTING UP

Audio			- PA stacks must be leveled, stabilized and gaffed or strapped together.

- Fold back wedges must not overhang stage.

- Cables on floor in walkways must be gaffed down to floor or covered

with cable covers.

- 2 crew to lift audio mixers where possible..

- RSD device to be used on PA and stage gear power.

- HPM power point tester to be used. Green & Red = OK

- All PA gear to be tested and tagged annually.

Lighting			- No ladders are carried or required by G.S.S.V.

- 2 crew are required to hang lights in awkward positions.

- Winch up trees must be locked off with locking bolts.

- Report all faults, no matter how small, to Glenn Smith.

- Allow plenty of ventilation for coils, do not cover with blacks.

- Cables in walkways to be gaffed down to floor.

- Be aware of wet risk, i.e. no lighting cans on front of stage where drinks might pill.

- Smoke machine must be approved for use by Venue Management and Glenn Smith.

- Lights must be focused away from audience eyes and ignition points.

General			- All equipment to be cleared in public areas before doors are opened.

- No running or jumping.

- No climbing past 1.5 m. high.

- Safety signs must be placed if appropriate.

-Reasonable care must be taken to avoid injury or damage to performers, their

equipment and the venue.

4. SHOWTIME

- People on stage for no reasonable purpose are to be politely escorted off.

- Venue management or security to be advised if the above procedure is refused by that  				   person.

- PA is usually set to target dance floor. Levels to suit event conditions.

- Earplugs available to crew if required.

5. PACKUP

- Pack up on stage only until guests/patrons leave.

- No heavy gear to be moved until guests/patrons have left.

- Lights must be cooled before handling.

6. LOADOUT

- As per load in.

Remember - Health and Safety is the overriding policy for Glenn Smith Sound & Vision.

Declaration.

I declare that I have read, understand and will follow the above procedures while working for Glenn Smith Sound & Vision on site at all times.

Name .............................................................

Signature.......................................................

Date...............................................................